~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

RCS version control information:

$Header: /srv/svn/cvs/cvsroot/arts/3rdparty/disort1.2/DISOTEST.doc,v 1.1 2006/02/21 16:23:28 olemke Exp $

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

DISORT Test Driver Documentation

There are two groups of DISORT test problems.  One group aims to test

ACCURACY, the other CONSISTENCY.

ACCURACY PROBLEMS

-----------------

The aim of the 'accuracy' problems is to compare the results with

answers obtained on an IBM 3081 computer running DISORT entirely

in double precision ( 14 significant digits ).  These problems are

not chosen to be comprehensive but rather to test extreme values

of the input parameters.  Hence, in cases where published tables

exist, only a few cases from the tables are examined.

NOTE:  When it is clear that a result should be zero -- for example,

the up-flux from a zero-albedo surface should be zero -- the

"correct" answer is set to zero.  In fact DISORT will usually

return a very small non-zero number in such cases, whose value

depends intimately on the computer being used.

In parentheses beneath each result is printed its ratio to the

"correct" answers (contained in DATA statements).  This ratio

is calculated by a small routine RATIO which protects against

under- and over-flow and attempts to return reasonable results.

If the ratio threatens to bump up against the largest or smallest

number representable on your computer, it will be set to that

number (roughly).  This can make the ratios differ from one

computer to the next.  However, most of the ratios should be

unity to at least a few significant digits.  If you run in less

than 14 digit precision, a few ratios will differ from unity,

and a few of those will just print as '*********' because you

will be comaring very small numbers which are sensitive to

precision, and when they differ by orders of magnitude their

ratio will not print in the supplied F format.  Some problems

were designed specifically to show this precision dependence,

so that the user has fair warning about running in low precision.

The worst degradation in accuracy from running in low precision

occurs for non-absorbing cases (single scattering albedo = 1),

because the eigenvalue problem becomes degenerate and very

sensitive to round-off errors in that case.

Test problems 1 to 5 involve single purely scattering layers

with successively more difficult phase functions.  Subcases are

for varying optical depths and single scattering albedos.  For

problems 1 and 3, Van de Hulst defines

reflectivity   = pi * U0(top) / incident flux

transmissivity = pi * U0(bottom) / incident flux

where U0 = azimuthally-averaged intensity.  We picked

incident flux = pi  so that our U0's are equal to Van de

Hulst's reflectivities and transmissivities.  DISORT reflected

and transmitted fluxes must be divided by the incident flux (pi)

to compare to the 'FLUX' column in Van de Hulst. (Get DISORT

transmitted flux by summing the direct and diffuse parts.)

Test problem 6 checks purely absorbing cases with an

increasingly complex mix of boundary and internal sources.

Solutions can be compared with exact analytic results.

Test problem 7 checks a general emitting/absorbing/scattering

medium with one computational layer and an

increasingly complex bottom reflectivity condition.

Test problem 8 checks an absorbing/isotropic-scattering

medium with two computational layers and isotropic incidence

at the top boundary.  Solutions can be compared to published

results.

Test problem 9 checks a general emitting/absorbing/scattering

medium with every computational layer different.  Cases A,B

can be compared to published results.  It also checks the special

case in PLKAVG where the two wavenumbers are close.

CONSISTENCY PROBLEMS

--------------------

The aim of the 'consistency' problems is to compare the results

obtained on two successive calls to DISORT, for identical

problem structures but taking differing paths through the program.

The answers obtained on the first call are saved and used to

calculate ratios after the second call.  Thus, ratios are not

printed after the first call (but answers are).

Test problem 10 checks that  USRANG = FALSE and  USRANG = TRUE

give identical results for the same set of angles.  It also

checks the special case branch for a 2x2 eigenvalue problem.

Test problem 11 checks that identical answers are obtained when

user levels in a single layer are replaced by computational levels.

Test problem 12 checks whether the shortcut for large absorption

optical depth (setting intensities to zero below a certain

absorption optical depth) is sufficiently accurate.  This is a

'worst case' problem in which a bottom boundary of albedo unity is

reflecting just below the cutoff level.  The user must decide

if he minds having large percent errors in intensities

which are negligibly small compared to the incident intensities.

Test problem 13 checks the option of determining albedo and

transmissivity for many sun angles at once.  Only fluxes are

involved.

NOTE:  The references cited in some of the test problems

can be found in the DISORT documentation file.

In the Garcia and Siewert tables ( cf. test problems 4

and 5 ) the sign of the zenith angle is reversed from the

convention used in DISORT.

** WARNING **

Every effort has been made to make the test problems

independent of each other, so that 'hangover' values from one

problem do not unintentionally become input to the next;

however, it is impossible to test all possible permutations

to be sure this is really true, and we could easily have

overlooked an isolated hangover or two.  Furthermore, for the

cases WITHIN a given problem, we make full use of hangover

values;  thus re-arranging these cases is almost certain to

lead to incorrect results.

Therefore, in modifying these test problems or devising

their own, users should:

(a) be very careful to set all input variables to DISORT before

calling it

(b) be aware of those few DISORT arguments that function as both

input and output, and not assume these retain their initial

values across multiple DISORT calls.